M.Tech II Semester Regular Examinations September, 2010

POWER SYSTEM CONTROL AND STABILITY

(ELECTRICAL POWER SYSTEMS)

- Using the classical model of one machine connected to an infinite bus obtain its equivalent circuit with necessary explanation.

- Derive the state space model of one machine system connected to an infinite bus for dynamic stability studies? What is the effect of excitation on dynamic stability?

- Construct a simplified linear model of excitation system. Derive the state space equations for the same.

- Determine the swing equation of a single generator connected to a double circuit line through the transformer and there from analyze the steady state stability. Give the comments.

- With the help of neat block diagram, analyze the effect of voltage and speed control of a synchronous machine with voltage regulation with one time sag.

- a) Write the methods to control the voltage stability.

b) List out various ways to analyze the voltage instability and collapse.

- A synchronous generator represented by a voltage source of 1.0pu in series with a transient reactance of j0.15 pu and inertia constant of 2.5MJ/MVA is connected to an infinite bus through a line of reactance of j0.3 pu. The infinite bus is represented by a voltage source of 1.0 pu in series with a reactance of j0.2 pu. The generator is supplying an active power of 1.0pu when a 3-phase fault occurs at its terminals. If the fault is cleared in 100milli seconds, determine system stability by plotting swing curve. Take t=0.05secs.

- a) Explain point by point method of solving swing equation.

b) Explain methods of improving transient stability.