M 080

MODEL PAPER

B.E. DEGREE EXAMINATION.

Second Semester

Aeronautical Engineering

IT 132 — ELECTRICAL ENGINEERING

(Common to Automobile Engineering/Production Engineering)

Time : Three hours	Maximum : 100 marks

Answer ALL questions.

PART A — (10  2 = 20 marks)

- State Thevenin's and Norton's theorem.

- Find the equivalent resistance between the terminals ‘a’ and ‘b’ in the resistive network shown in Fig. 2.

Fig. 2

- Give atleast four comparisons between electric and magnetic circuits.

- Tow long parallel conductors lies in air at 0.5 m apart from each other and carries a current of 2 amps each in the same direction. What is the magnitude and direction of force experienced by both conductors?

- Find the form factor of the periodic waveform shown in Fig. (5).

Fig. 5

- A series RLC circuit has Q–factor of 75 and bandwidth of 160 Hz. Calculate the resonant frequency and lower and upper half power frequencies.

- What is back emf in a d.c. motor? What is its significance?

- State the necessary conditions for voltage build–up in a dc shunt generator.

- A 230/460 V transformer has a primary resistance of 0.2  and reactance of 
0.5  and corresponding values for secondary are 0.75  and 1.8  respectively. Find the secondary terminal voltage when supplying 10 A to load at 0.8 pf lagging.

- Mention atleast three characteristic features of synchronous motor.

PART B — (5  16 = 80 marks)

- (i)	State and explain Kirchoff's circuital laws.	(4)

- (ii)	Determine the magnitude and direction of current in the detector (galvanometer) whose resistance is 10  in the bridge circuit shown in 
fig (11. ii) using mesh current analysis.	(12)

Fig. (11. ii)

- (a)	(i)	Explain the term leakage flux and fringing in magnetic circuits.	(4)

- (ii)		The rectangular magnetic core shown in Fig. 12 (a. ii) has an air 
		gap cut across one of its limbs. Find the exciting current needed to 
		create an air gap flux of 2 mWb. The relative permeability of the 
		core is 4000.			(12)

Fig 12 (a. ii)

Or

- (b)	(i)		A coils having a self inductance of 120 H is connected in series 
		with a second coil having a self inductance of 180  and the 
		combination is found to have an effective inductance of 
		140 Calculate (1) mutual inductance, (2) the coefficient of 
		coupling (3) effective inductance of the series combination if 
		terminals of one of the coils are interchanged.	(8)

- (ii)		Derive an expression for lifting power of a magnet.	(8)

- (a)	(i)	A resistor of 120  is connected in series with a  60 F capacitor 
			across a 240 V, 50 Hz supply. Find (1) Impedance,  (2) The current  
			(3) Power factor (4) Phase angle (5) Voltage across the resistor and 
			capacitor. Draw the phasor diagram. 	(8)

- (ii)		In a RLC series circuit, if w1 and w2 are half power frequencies 
		d if wr is the resonant frequency prove that wr2=w1 w2.	(8)

Or

- (b)		(i)		A balanced star–connected load of (8+j 6) Ω per phase is connected 
		to a three phase, 230 V supply. Find (1) line current (2) power factor 
		(3) active power (4) reactive power and (5) total volt ampere.	(6)

- (ii)		Derive an expression for wattmeter readings and power factor for 
		three phase power measurement by two wattmeter method.	(10)

- (a)	(i)	Derive an expression for e.m.f. developed in a d.c. generator.	(8)

- (ii)		A d.c. shunt generator delivers 195 Amps at a terminal voltage of 
		250 V. The armature resistance and shunt filed resistance ae  0.2  
		and 50  respectively. The iron and friction losses equal 950 W. 
		Find (1) emf generated (2) Cu. losses  (3) Input power and 
		(4) efficiency.			(8)

Or

- (b)	(i)		Describe the different characteristics of a d.c. shunt and series 
		motors with graphs.			(10)

- (ii)		A dc	shunt motor runs at 900 rpm from a 460 V supply when 
		taking an armature current of 25 A. Calculate the speed at which it 
		will run from a 230 V  supply when taking an armature current of 
		15 A. The resistance of the armature circuit is 0.8 . Assume the 
		flux per pole at 230 V to have decreased to 75% of its value 
		at 460 V.			(6)

- (a)	(i)	Explain with diagrams how a transformer works under load 
	condition.			(8)

- (ii)		A 50 Hz, 8 pole induction motor has a full–load slip of 4%. The rotor 
		resistance is 0.001 /phase and the standstill reactance is 
		0.005 /phase. Find the ratio of maximum to full load torque and
		the speed at which the maximum torque occurs.	(8)

Or

- (b)	(i)		Calculate the speed and open circuit line and phase voltages of 
		a 4 pole, 3 phase, 50 Hz star connected alternator with 36 slots and 
		30 conductors per slot. The flux per pole is 0.05 Wb sinusoidally 
		distributed.			(10)

- (ii)		Write a short note on V–curves of a synchronous motor.	(6)

—————