ANNA UNIVERSITY, CHENNAI 25

### MODEL QUESTION PAPER

B.E. Aeronautical Engineering, VI SEMESTER

# AE 342 - PROPULSION-II

TIME – 3 Hours						MAXIMUM : 100 Marks

PART A                     (10 X 2 = 20 Marks)

ANSWER ALL QUESTIONS

1.	Explain the difference between Impulse and reaction blade.

2.	How will you classify turbine blade cooling?

3.	Draw T-S diagram of Ramjet Engine.

4.	Name the components in Pulse Jet Engine.

5.	Describe briefly three important application of rocket propulsion

6.	Define specific propellant consumption.

7.	What are the types of propellant injector?

8.	Name any two properties of liquid propellant.

9.	Define thrust coefficient.

10.	Explain the principles of Nuclear propulsion?

# PART B                    	           (5 X 16 = 80 Marks)

11	i.	Define degree of reaction and prove for 50% of reaction α = β and αe = βi

suffix i.e. stands for inlet and exit.					               8

- At the mean diameter of a gas turbine the blade velocity is 350 m/s. The blade

angles at inlet and exist are 200 and 540 respectively, and the blades at this

section are designed to have a degree of reaction of 50%. The mean radius of the

blades is 0.216m and the blade mean height is 0.07m. Assuring the blades are

designed according to vortex theory calculate.

The flow velocity.								    3

The angles of blades at the tip and at the root.					    3

The degree of reaction at the tip and at the root of the blades.			    2

12 (a)		A ram jet engine operates at M = 1.5 at an attitude of 6500m. The diameter of the

inlet diffuser at entry is 50 cm and the stagnation temperature at the nozzle entry

is 1600K. The calorific value of the fuel used is  40 MJ/Kg. The properties of the

combustion gases are same as those of the air [ r = 1.4, R = 2875 / Kgk ]. The

velocity of the air at the diffuser exit is negligible. Calculate

    - The efficiency of ideal cycle						    2

    - Flight spaced								    1

    - Airflow rate 								    2

    - Diffuse, pressure ratio							    2

    - Fuel air ration								    1

    - Nozzle pressure ratio							    1

    - Nozzle Jet mach number						    2

    - Propulsive efficiency							    3

    - Thrust									    2

Assume the following values

ηD = 0.90	 ηB = 0.98	ηj = 0.96

Stagnation pressure loss in combustion chamber = 0.02 PO2

At z	= 6500m the properties of air are

T1 = 245.90 K	P1 = 0.44 bar

a1 = 314.50 m/s	S1 = 0.624 kg/m3

( OR )

(b)	i.	With aid of a neat diagram, explain the working principle of ram jet engines     8

- Draw the thermodynamic cycle of ram jet engine and derive the equation

for thrust.									   8

13  (a).	Derive the following relation for the characteristic velocity 				   8

ao

(i)         V* =

(γ + 1) /2( γ – 1)

γ      2

γ + 1

————

√

R T0 M

- V* =

(γ + 1) /2 (γ – 1)

2

√ γ

γ + 1

Calculate the values of the characteristic velocity for				     8

i.	γ = 1.25	M = 18.3	and	T0 = 3300 K

ii.	γ = 1.33	M = 29.0	and	T0 = 3508 K

( OR )

- A rocket flies at 10,080 km/hr with an effective exhaust jet velocity of

1400 m/s and propellant flow rate 5 Kg / s. If the head of reaction of the

propellant is 6500 KJ / Kg of  the propellant mixture determine

(i)	Propulsion efficiency and propulsion power 			 6

(ii)	Engine output and thermal efficiency           			 6

(iii)	Over all efficiency                                        			 4

14 (a)	I	Explain briefly the following terms in solid propellant rocket

(i)	Linear burning rate 						 2

(ii)	Combustion rate     						 2

(iii)	Propulsion area ratio 						 3

(iv)	Equilibrium combustion pressure 				 2

(v)	Cigarette burning   						 3

II	Explain briefly what is meant by

(i)	Mono propellants						 2				(ii)	Hypergolic propellant						 2

( OR )

(b)	i.	Draw a neat sketch of a Turbo pump feed system employed in rockets   	 8

ii. 	How does cavitation occurs in this system and what measures are taken to

prevent it 									 8

15 (a)	i	How is regressive, neutral and progressive burning of the solid

propellant grown achieved? Explain with the aid of diagram.    		 8

ii.	State 5 advantages and 5 disadvantages of solid propellant rocket   		 8

( OR )

(b)		Draw neat sketches, explaining the general working of the following rocket engines.

(i)	Hybrid engine                                                          				 8

(ii)	Electrical rocket                                                        				 8