ANNA UNIVERSITY, CHENNAI 25

### MODEL QUESTION PAPER

B.E. Aeronautical Engineering, , VI SEMESTER

# AE344 - THEORY OF VIBRATIONS

TIME – 3 Hours						MAXIMUM : 100 Marks

PART A                     (10 X 2 = 20 Marks)

ANSWER ALL QUESTIONS

1.	Given a sinusoid signal with frequency equal to 2 rad/s and initial velocity equal to 5 units / sec,  find x (t).

2.	What must be the length of a simple pendulum  for the period of oscillation to be equal to 1 second?

3.	What is a viscous damper?

4.	A system is described by the following differential equation:

d2X                  dX

- ---------   +   c   --------  +   2000X = 0

dt2                     dt

Find the value of c if the system is critically damped.

5.	Name the parameters that govern the natural frequency of longitudinal vibrations for a given bar.

6.	With the example of a  simple pendulum vibrating system, write a short note on the meaning of linearization.

7.	List a few approximate methods that can be used to calculate the natural frequencies of a given system.

8.	Explain the term AEROELASTICITY

9.	Sketch COLLAR'S triangle and mention its implication.

10.	Write a short note on Mass Balancing.

PART B - (5 x 16 = 80 marks)

11.	(i)	Define simple harmonie motion and give an illustrative

example of the same.								6

(ii)	For the system shown in Figure 1 below, determine the natural

frequencies and eigen vectors. Given : M1 = 2kg, M2 = 4 kg,

K1 = 40 N/m,  K2 = 100 N/m, K3 = 200 N/m.					10

Figure 1

12 (a)		The system shown in Figure 2 is given an excitation at a frequency w. The

absolute value of the response = 3.6mm. Given m = 0.05 kg ,  K = 45 N/m ,

and  y (t)  = 2 sin (wt) mm, determine w.					16

Figure 2

( OR )

12 (b)	(j)	Explain the meaning of RESONANT FREQUENCY.			6

(ii)	For  a SDOF  spring - mass system oscillating in the vertical plane,

obtain the relationship between natural frequency and

static deflection.								8

(iii)	Give an example of a MDOF System						2

13 (a)	(i)	Differentiate between principal coordinates and

generalized coordinates							6

(ii)	State Hamilton`s Principle							4

(iii)	Write notes on the Lagrangian equations and their application		6

( OR )

13 (b)		Derive and obtain the governing differential equation for a uniform beam

undergoing free lateral vibrations. Treate the beam as a CONTINUOUS

system.								          16

14 (a)	(i)	State the D`Alembert Principle						4

(ii)	With an example, explain the orthogonality condition in

vibration theory								6

(iii)	Name any one vibration measuring instrument and explain

now it functions.								6

( OR )

14 (b)		The torsionally geared system shown in Figure 3 consists of 3 discs

with  moments of inertia I1 = 4600kg cm2 , I2 = 600kg cm2 and

I3 = 80kg cm2 respectively. The gear ratios are n2 = 2 and n4 = 4

with respect to I1. Find the natural frequencies of the system. 		16

K1 = 35 x 106 Ncm / rad

K2 = 15 x 106 Ncm / rad

K3 =  5 x  106 Ncm / rad

Figure 3

15 (a)	(i)	Define the following: elastic axis, aerodynamic centre, flexural axis,

aileron reversal speed.							         8

(ii)	For the simple 2-D case, derive and obtain an expression for the wing

divergence speed.							         8

( OR )

15 (b)		An airfoil is undergoing contrained vertical translatory motion and

rotational motion in the same plane. Derive and obtain the governing

differential equations of motion for the airfoil. The springs

associated with the translation and rotational motions have spring

constants Kn and Ka respectively.       		        		      16