M 040

MODEL PAPER

B.E. DEGREE EXAMINATION.

Fourth Semester

Automobile Engineering

AT 235 — AUTOMOTIVE PETROL ENGINES

Time : Three hours	Maximum : 100 marks

Answer ALL questions.

PART A — (10  2 = 20 marks)

- List out eight important parts of 4 stroke petrol engine.

- Write the firing order of four cylinder and six cylinder engine and its significance.

- List out the functions of carburetion.

- Sketch the cut–sectional view of Electrical Pump.

- List four required properties of a good coolant.

- What are the various types of lubrication systems?

- Mention four important engine variables on Engine–knock.

- Sketch four types of combustion chamber.

- Name various types of two stroke engines.

- List out various types of scavenging pumps.

PART B — (5  16 = 80 marks)

- Compare by way of tabulation the differences between four stroke and two stroke engine.

- (a)	Sketch and explain the working principle of carburettor involving starting,  idling and acceleration circuits.	(10)

Or

- (b)	A petrol engine consumes 6.8 kg of petrol per hour. The specific gravity of the fuel 0.7 and its temperature 32C. The air fuel ratio is 15. The choke tube has a diameter of 20 mm. Calculate the diameter of the single jet of the carburettor if the top of the jet is 5 mm above the petrol level in the float chamber. R of air can be assumed.	(6)

- (a)	(i)	Why we need a cooling system in Engines?	(6)

- (ii)	Sketch and explain Mist lubrication system.	(10)

Or

- (b)	(i)	List the essential properties of lubricant used in vehicles.	(6)

- (ii)	Explain with sketch the thermosyphen  cooling system.	(10)

- (a)		Sketch and explain various stages of combustion in SI engines.

Or

- (b)	(i)	Define the term knock and abnormal combustion.	(6)

- (ii)	List the factors controlling combustion chamber design.	(10)

- (a)	Sketch and explain the working of any two types of two stroke engine.	
					(2  8 = 16)

Or

- (b)	What are the various scavenging methods? (sketch and explain).

——————