M 075

MODEL PAPER

B.Tech. DEGREE EXAMINATION.

Fourth Semester

CHEMICAL ENGINEERING

CH 242 — Physical Chemistry

(Common to Textile and Leather Technology)

Time : Three hours	Maximum : 100 marks

Answer ALL questions.

PART A — (10  2 = 20 marks)

- How much current is needed to deposit 2 g of chromium metal from a solution of Cr3+ in a period of 50 minutes?

- What is the potential of a lead electrode that is in contact with a solution of 0.02 M in Pb2+ ions? The standard electrode potential (E∘) for

- Pb 		Pb2++2e− is equal to 0.13 volt.

- How is the phase diagram of a solid solution differs from the eutectic phase diagram?

- How many number of phases, components and degrees of freedom are available in the following equilibrium?

- CaCO3 (s)                   CaO(s)+CO2 (g)

- How are colloidal metal and metallic sponge catalysts prepared?

- Suggest a catalyst for the following reactions :

- (a)	C2H5OH 		C2H4O+H2

- (b)	C6H5CH2OH             	C6H5CH3

- (c)	n–Heptane		Toluene

- (d)	Vegetable Oil 	Fat

- What happens when solutions of sodium nitrate, barium nitrate and aluminium nitrate are separately added to 50 ml each of arsenic sulphide sol?

- Distinguish between a dark reaction and a photochemical reaction.

- What is called resonance fluorescence?

- Define energy of activation.

PART B — (5  16 = 80 marks)

- (i)	Distinguish between physical adsorption and chemisorption. Discuss the characteristics of chemical adsorption. Explain an adsorption isotherm to reflect the characteristics of physical adsorption.	(8)

- (ii)	Account for the low quantum yield in H2−Br2 reaction.	(8)

- (a)	(i)	Write briefly on gaseous actinometer.	(8)

- (ii)	Discuss the kinetics and mechanism of the photochemical reaction between Hydrogen and Chlorine.	(8)

Or

- (b)	(i)	State Beer’s law and mention its applications.	(8)

- (ii)	A uranyl oxalate actinometer was irradiated for 20 minutes with a light of wavelength 4400 Å. At the end of this time, it was found that oxalic acid equivalent to 15 ml of 0.001 M potassium permanganate was decomposed by light. The quantum yield of this reaction was 0.52. Find the average intensity of light used in ergs per sec.			(8)

- (a)	(i)	Discuss the construction and working of the Hydrogen electrode.	(8)

- (ii)	What is the potential of a copper–silver cell if the Ag+ concentration is 1  10−3M and the Cu2+ concentration is 
1  10−4M. The standard potential of the cell is 0.46 V and the cell reaction is

- 2Ag(aq)++Cu(s)	2Ag(s)+Cu(aq)2+	(8)

Or

- (b)	(i)	Derive Nernst equation and explain its significance.	(8)

- (ii)	The equivalent conductance of NH4Cl at infinite dilution is 
147.7 ohm−1cm2eq−1 and the ionic mobilities of hydroxyl and chloride ions are 198 and 76.3 respectively. Calculate the equivalent conductance of NH4OH at infinite dilution.	(8)

- (a)	(i)	Explain the catalytic functions of the following :

- (1)	Metals

- (2)	Semiconductors

- (3)	Insulators.			(8)

- (ii)	What are carriers? How are they classified? Explain their catalytic functions with examples.			(8)

Or

- (b)	(i)	Account for the following :

- (1)	Adsorption is an exothermic process.

- (2)	In permanganimetric titration against oxalic acid, the first few drops of potassium permanganate takes more time for decolorisation.

- (3)	Arsenic oxide present in the platinised asbestos inhibit the rate of formation of sulpuric acid.

- (4)	Heterogeneous catalysis is the most widely used technique in industry.			(8)

- (ii)	To which type of colloidal system, the following belong :

- (1)	Cu–Ni alloy 	(4)	Mist

- (2)	Butter	(5)	Latex

- (3)	Milk of Magnesia	(6)	Smog	(8)

- (a)	(i)	Derive an expression for the second order rate constant for a reaction in which the reactants have the same initial concentration?	
 				(8)

- (ii)	What is B.E.T. equation? Explain its significance.	(8)

Or

- (b)	(i)	Discuss the principle behind the Pattinson’s process of desilverisation of lead.			(8)

- (ii)	Explain the term fractional distillation and mention its industrial utility.			(8)

——————