MODEL PAPER

B.TECH DEGREE ( CHEMICAL ENGINEERING) ( 5th SEMESTER)

FLUID MECHANICS (CH – 333)

Time :  3 hrs.								Max. Marks : 100

# PART -  A   ( 10 X 2 = 20 Marks)

- When do you prefer an inclined manometer ?

- A vaccum gage reads 10 in.  Hg when the atmospheric pressure is 30 in. Hg.  Assuming the density of mercury to be 13 . 595  kg / m3 , determine the pressure in Pascal.

- Define the following dimensionless numbers:

  - Weber number  (ii)    Froude Number, (iii)    Euler’s Number

- Explain geometric similarity, kinematic similarity and dynamic similarity.

- Distinguish between Laminar and turbulent flow.

- Distinguish between kinetic energy correction factor and momentum correction factor.

- Define sphericity factor.

- Water is flowing in two different pipes of diameters ‘d’ and ‘D’ .  Sketch the friction factor vs Reynolds Number for the two cases.

- Define hydraulic radius and equivalent diameter.

- Why is there a negative sign in the pressure – height relation?

# PART -  B ( 5 X 16 = 80 Marks)

- The density of a sample of cement is found to be 3100 kg/m3.  25.4 gm of this cement is packed into a column of diameter = 2.50 cm to form a bed of 27.3 cm thick.  When a pressure drop of 12.5 cm of Hg is established across the bed, it is found that 774 secs.  are required to drive 250 c m3 of air through the bed.

Calculate the average diameter of particles in the cement (assuming spherical) and the surface area per gram of cement.

Data  :  µair  = 1.83 X 10-5 kg / m – sec2         ςHg  = 1.355 X 104  kg/m3

g = 9.81  m/sec2

12.   (a)  A uniform film of oil 0.13mm thick separates two dics, each of 200mm               diameter, mounted coaxially. Ignoring edge effects, calculate the torque                necessary to rotate one disc relative to other at a speed of 7 revolutions/sec., if the oil has viscosity of 0.14 Pascal – sec.

(OR)

(b) Compute the atmospheric pressure at elevation 6000m, for the two cases listed below.  Considering the atmosphere as a static fluid.

Case (i)    Constant temperature between sea level and 6000m and

Case (ii)    air temperature decreasing linearly with elevation at a standard rate of

0.0065º c/m.

Data :  At sea level T = 15ºc  P = 100 KN/m2√  = 12.03 N/m3

13.  (a) Bring out the importance of the relation between the shear stress and velocity

gradient in characterizing the behaviour of flowing fluids.

- Water at room temperature flows through a smooth straight pipe A of internal  diameter 4cm at an average velocity of 42 cm/sec.  Oil flows through another pipe B of internal diameter 12.5 cm at such a velocity that dynamic similarity exists between the two streams.  Calculate the velocity through pipe B.  (Specific Gravity of oil = 0.85, viscosity of oil = 2 cp and viscosity of water = 1 cp.)

(OR)

(c )   Derive Bernoulli’s equation for flow through a straight pipe.

(d )  Explain the significance of each term in the above equation

14.  (a)  Derive Hagen Poisuille equation for pressure drop in laminar flow.

(OR)

The power required by an agitator in a tank is a function of the following four  Variables  : (a) diameter of the impeller, (b)  RPM of the impeller, (c ) viscosity of liquid, (d) density of liquid and acceleration due to gravity (g)

From the dimensional analysis, obtain a relation between power and the variables. The power of consumption is found, experimentally, to be proportional to the square of the speed of rotation.

- (a)  A two dimensional fluid motion is specified in the Legrandian manner by the equation x =  x0 ekt ,  y = y0 e-kt

  - find the path of fluid particle

  - the expression for velocity in the Eulerian form

  - state whether the motion is steady and

  - whether it is kinematically possible for an incompressible fluid

(OR)

(b)  For the velocity profile u= umax  ( I – r/rw) 1/n  compute the kinetic energy correction factor and momentum correction factor. What are the values when n = 7?

----