M 046

MODEL PAPER

B.E. DEGREE EXAMINATION.

Fourth Semester

Electronics and Instrumentation Engineering

EI 235/IT 237 — TRANSDUCER ENGINEERING

(Common to Instrumentation and Control Engineering and 
Instrumentation Engineering)

Time : Three hours	Maximum : 100 marks

Answer ALL questions.

PART A — (10  2 = 20 marks)

- Why is periodical calibration of measuring instrument necessary?

- List any two sources of systematic error in a measuring instrument.

- What is the difference between accuracy and precision?

- What is the 5% settling time in the step response of a first–order instrument with a time–constant of 12 sec.?

- What is meant by self–heating error in a resistance thermometer?

- Why is dynamic compensation network used with hot–wire anemometers?

- Mention the problem that occurs when phase insensitive demodulator is used with LVDT.

- Sketch the desired frequency response of a capacitor microphone.

- Give the material and sensitivity of IC temperature sensor.

- Mention two advantages of smart sensor.

PART B — (5  16 = 80 marks)

- (i)	Define standard.  How are standards classified?  Give the international standards used for mass, length, and time.	(8)

- (ii)	Ten measurements of a pressure made by an instrument at different time intervals give the following readings in kg/cm2 :	(8)

- 7.92, 8.03, 8.10, 7.95, 8.01, 7.98, 7.95, 8.07, 7.94 and 7.97.

- Find arithmetic mean, standard deviation, and most probable error.

- (a)	(i)	Define sensitivity and resolution of a measuring instrument.	(4)

- (ii)	What are the three ways of specifying the non–linearity of an instrument?			(4)

- (iii)	A first–order thermometer with a time–constant of 5 sec is used to measure the temperature of a furnace fluctuating between 540C and 580C in a sinusoidal manner.  The frequency of fluctuation is 0.04 Hz.  Find the maximum and minimum readings of the thermometer.			(8)

Or

- (b)	(i)	Derive an expression for the output of a first–order instrument when subjected to unit impulse input.  Also sketch the response.	(8)

- (ii)	The gas pressure inside a vessel starts increasing from the initial value of 100 kPa at the rate of 2000 Pa/sec.  The pressure is measured by a second–order instrument with a damping ratio of 0.7 and a natural frequency of 30 Hz.  Sketch the input and output variation with time and find the steady–state error in instrument reading after transient in output has vanished.	(8)

- (a)	(i)	Give the construction details of bonded and unbonded strain gauges.  Explain any two applications of bonded strain gauges.	(8)

- (ii)	A resistance potentiometer with a 20 mm travel has a resistance of 10,000 ohms.  It is connected to a 10 V d.c. source with negligible internal resistance.  Calculate the actual displacement when the output of the potentiometer is 5 V.  The resistance of the voltmeter is 20,000 Ohms.			(8)

Or

- (b)	(i)	Explain with a sketch the construction details and characteristics of a piezo resistive pressure transducer.	(8)

- (ii)	Describe the temperature Vs resistance characteristic of a thermistor with equation.  Explain any one method of linearizing thermistor characteristic.		(8)

- (a)	(i)	Explain with a sketch the principle of operation of a variable reluctance accelerometer.		(8)

Movable

Conducting Plates

Fixed

x

(ii)	Five square plates of area 16 cm2 (each) are arranged as shown in Fig. 14(a).  The plate spacings are 0.2 mm (each).  The arrangement is to be used as a displacement transducer by observing change in capacitance with distance x.  Calculate the sensitivity of this device in pFcm.			(8)

Fig. 14(a)

Or

- (b)	(i)	Draw the input–output characteristic of LVDT.  Explain the phase adjustment circuits and null–reduction technique used for LVDT.	(8)

- (ii)	Explain with a sketch the construction details, principle of operation and signal conditioning circuits of a capacitive transducer used for differential pressure measurement.	(8)

- (a)	(i)	Define the two sensitivity constants of piezo electric transducer.  Derive the relation between the two constants.  Also draw the equivalent circuit of piezoelectric transducer.	(8)

- (ii)	Explain with a sketch the working of an encoder that gives 4–bit digital output proportional to linear displacement.  Mention the advantages of digital transducers.	(8)

Or

- (b)	(i)	Define magnetostrictive effect.  Name two materials used in magnetostrictive transducers.  Describe any one application of magnetostrictive transducer.		(8)

- (ii)	Explain with a sketch the working of a fiber–optic displacement transducer.  Draw its input–output characteristic.	(8)

———————