MODEL QUESTION PAPER

# B.E. INSTRUMENTATION AND CONTROL ENGINEERING

SEMESTER V

# IL331 – NETWORK ANALYSIS AND SYNTHEIS

Time : 3 Hours 							Max Marks : 100

# Answer All Question

# PART – A  (10 x 2 = 20 marks)

1.	Define poles and zeros of a Network Function and plot them in a complete plane for

N(s)=4(s+1)(s2+4s+20).

2.	A two port network is characterized by V1 = 10I1+5I2 and V2 = 5I1+12I2. Find the transmission parameters A and C.

3.	State any three properties of RC driving point impedance function.

4.	Check whether the function F(s)=3s2+5s(s2+1) is positive real or not.

5.	Find whether the impedance function Z(s)=K(s2+1)(s2+9)(s2+2)(s2+10) is an LC.

6.	Draw any one shape of configuration of synthesis for the immittance function F(s)=2(s+1)(s+3)(s+2)(s+6)

7.	Define zeros of transmission and their properties.

8.	What are the two basic types of constant – resistance networks and their important properties?

9.	What is the use of frequency transformation in filter design?

10.	Compare Butterworth filter with chebyshev filter.

PART – B (5 x 16 = 80 Marks)

11.	An impedance function has simple poles at-1 and –4 and simple zeros at –2 and –5 z(0)=10.

Synthesize the impedance function in a foster form and a causer form.	         (8+8)

12.a)	For the network shown in Figure 12(a), show that y11=(s+1)(s+3)(s+2)(s+4) and −y12=k(s+1)(s+2)(s+4). Determine the value for K.

Figure 12(a)

OR

12.b)	For the network shown in Figure 12(b), find the transmission parameters and the h-parameters.

Fig.12.(b)

13.a)i)	Explain the terms “Casuality” and “Stability” in network synthesis in time domain and frequency domain.							         (4+4)

ii)	Test the following polynomial for the Hwuritz property.

s7+2s6+2s5+s4+4s3+8s2+8s+4								 (8)

## OR

13.b)	Determine whether the following functions are positive real.

(i)	F(s)=s+2s2+3s+2							(3)

(ii)	F(s)=3s2+5s(s2+1)							(5)

(iii)	F(s)=(s2+2s+25)(s2+5s+16)							(8)

14.a)i)	Describe the synthesis of lattice networks terminated in a load of constant resistance R.											 (8)

ii)	Synthesis the lattice network terminated in a 1-ohm resistor to meet the specification

Z12=s2−s+1s2+s+1.

OR

14.b)	For the networks shown in Figure.14(b), show that

(i)	V2V0=1(2+y)	   							 (6)

(ii)	Synthesize Y when V2V0=0.5(s2+2)(s2+2s+2)       			           (10)

Fig.14.(b)

15.a)i)	Synthesize a third order Butterworth low pass filter terminated in 1-ohm resistance and having transfer impendence Z21 of third Butterworth polynomial.	           (10)

ii)	Hence synthesize the changes in the above filter when it is terminated in a load of 600 ohm instead of 1-ohm.							             (6)

OR

15.b)	Describe in detail the design of high pass filter and band pass filter from the design of low pass filter using frequency transformation.				         (8+8)

-----