ANNA UNIVERSITY :: CHENNAI – 600 025

MODEL QUESTION PAPER

V SEMESTER

B.TECH. INFORMATION TECHNOLOGY

# IF351 – DIGITAL SIGNAL PROCESSING

Time : 3 Hours					                               Max. Marks : 100

Answer all Questions

PART – A (10 X 2 = 20 MARKS)

- State and prove the convolution property of Z transform.

- Check the system is linear or not  y(n) = x(n)+ay(n-1)

- Write equations for finding DFT and IDFT using Z transform.

- Draw the radix 2 butterfly structure for DIF

- Draw the implementation for the generalized for IIR filter using direct form II.

- Explain how the addition and multiplication of (H1, H2) impulse responses implemented in filter design

- Write equations for Hanning and Blackman window.

- Why frequency prewarping procedure is adopted in the design of IIR filter?

- Write two advantages of musical sound processing and briefly explain.

- Explain the effects due to upsampling.

PART - B (5 x 16 = 80 Marks)

11.i)	The impulse response of a linear TI system is h(n) = {1, 0, 1, -1}. Find the response of the system to the input signal x(n) = {1, 0, 2, 1}.

ii)	Check whether the system y(n) = x(n) – x(n-1) is LTI and stable.

12.a)	Develop and draw the 8 point radix-2 DIT FFT algorithm for DFT computation.

(OR)

12.b)	Compute the DFT of the following sequence

x(n) = 0     0 n  2

= 1     3 n  6

= 0     n=7

Plot magnitude and phase spectra

13.a)	Design a LPF with following specifications. Use Hamming window and at least 8 points.

H(eiw)

1

/2

-/2

0

(OR)

13.b)i)	Obtain H(z) from H(s) when T = 1 sec.

ii)	Design a digital BPF using w1 & w2 as cutoff frequencies

14.a)i)	Perform the following using Floating Point arithmetic.

1.5 x 1.75 and 1.5 x 1.75

ii)	Realize the following H(z) given by

using cascade and Parallel form with Direct form-I.

(OR)

14.b)i)	What is meant by quantization error?  Explain briefly.

ii)	Realize the following filter using cascade technique with DF-I and DF-II.

15.a)	Briefly explain

  - Interpolator

  - Decimator

  - Effects due to sampling rate conversion

(OR)

15.b)i)	Write a note on Musical sound processing

ii)	Explain how the data compression is achieved in speech signal and discuss a technique to check the quality.

******