M 051

MODEL PAPER

B.E. DEGREE EXAMINATION.

Fourth Semester

Instrumentation Engineering

IT 240 — INSTRUMENTATION COMPONENTS

Time : Three hours	Maximum : 100 marks

Answer ALL questions.

PART A — (10  2 = 20 marks)

- List the various types of Instrumentation components.

- State the relative merits of pulleys and gears.

- Bring out the difference between Gate valve and diaphram valve.

- List out the advantages and disadvantages of pneumatic relays.

- Define Power rating of a resistor.

- List the types of fixed resistors.

- Explain the function of LDR.

- With neat diagram explain the principle of opto coupler.

- Define the term ‘transformation ratio’ of a transformer.

- Bring out the difference between relays and switches.

PART B — (5  16 = 80 marks)

- (i)	What are the types of gears? Explain how tight gearing without backlash can be achieved.			(6)

- (ii)	Explain the principle and applications of Geneva mechanism.	(6)

- (iii)	With neat diagram explain any two types of mechanical Indicating Mechanisms.			(4)

- (a)	(i)	With neat diagram explain the construction and working principle 
	of flapper nozzle.			(6)

- (ii)	Discuss its response curve.		(3)

- (iii)	What are the factors to be considered for selecting a control valve.	(7)

Or

- (b)	(i)	With neat diagram explain the operation of pneumatic actuator 
	with and without positioner.		(10)

- (ii)	Discuss the disadvantages of pneumatic actuator without positioner and also explain how it is eliminated in with positioner scheme.	(6)

- (a)	With neat diagram explain the construction, working principle and application of a magnetic amplifier.		(6 + 8 + 2)

Or

- (b)	(i)	Explain the construction of a variable reluctance stepper motor.	(4)

- (ii)	Derive the torque equation of the stepper motor.	(8)

- (iii)	Explain the application of stepper motor in open loop mode.	(4)

- (a)	(i)	Explain the construction and characteristics of thermistor.	(6)

- (ii)	Explain the application of thermistor in

- (1)	Level measurement.

- (2)	Temperature Compensation.	(10)

Or

- (b)	(i)	With neat diagram explain the construction and working principle of SCR.			(10)

- (ii)	Explain how SCR is used in power control application.	(6)

- (a)	(i)	Explain the construction, working principle and application of PIN 
	diode.			(6)

- (ii)	Bring out the difference between Dispersion and Reflection prism.		
				(5)

- (iii)	Explain the working of LED.		(5)

Or

- (b)	(i)	Explain the term grating and discuss its application.	(8)

- (ii)	Write short notes on resonators and reflectors.	(8)

———————