M 057

MODEL PAPER

B.E. DEGREE EXAMINATION.

Fourth Semester

Mechanical Engineering

ME 235 — PRODUCTION TECHNOLOGY

Time : Three hours	Maximum : 100 marks

Answer ALL questions.

PART A — (10  2 = 20 marks)

- Name any two moulding sand varieties that are often used in steel foundries.

- What is a core print?

- Define maximum draft in moulding.

- Write the formula for finding out the maximum bending force, P, excluding friction.

- E60XX  is the designation for a mild steel–coated electrode. What does ‘E’ stands for?

- Name any two factors that affect the weld zone in friction welding.

- How is the pressure required to form the metal obtained in explosive forming process?

- Which is the source of energy in electro–hydraulic forming process?

- Name at least two common thermoplastics.

- What is called wet plastic extrusion?

PART B — (5  16 = 80 marks)

- (i)	Describe the difference between expendable and permanent moulds.

- (ii)	Give the step–by–step procedure of investment casting and die–casting processes.

- (a)	(i)	What are the types of deflection that rolls under go? Explain them.

- (ii)	Explain the stages in forging a connecting rod of an internal combustion engine.

Or

- (b)	(i)	Compare extrusion and drawing processes.

- (ii)	Discuss the effect of various processing parameters in shearing.

- (a)	 (i)	Show and explain the features of neutral, reducing and oxidizing flames with their respective applications.

- (ii)	Describe the working of laser beam welding process with its applications. What are the advantages of it over electron beam welding?

Or

- (b)	(i)	What are fusion welding and solid state welding?

- (ii)	List the factors that contribute to the development of cracks in welds and explain their formation and the preventive measures that are available.

- (a)	(i)	How does explosive forming process different from other conventional metal forming processes?

- (ii)	With a schematic illustration explain how a cup shape is obtained using explosive forming process.

Or

- (b)	(i)	What are the advantages and disadvantages of high energy rate forming process?

- (ii)	What is the basic principle of electro–hydraulic forming process? Explain the working of both capacitor discharge through a gap and a wire processes.

- (a)	(i)	List the properties of plastics for their wide engineering applications.

- (ii)	Describe the process of extrusion of plastics using screw type machine.

Or

- (b)	(i)	What are the advantages and disadvantages of Ram type of plastic extrusion machine?

- (ii)	Compare compression and transfer moulding processes.

———————