M 076

MODEL PAPER

B.E. DEGREE EXAMINATION.

Second Semester

Mechatronics Engineering

EC 153 — ELECTRONIC DEVICES AND CIRCUITS

Time : Three hours	Maximum : 100 marks

Answer ALL questions.

PART A — (10  2 = 20 marks)

- Give the value of energy gap for semiconductors and insulators.

- What is meant by extrinsic semiconductor?

- Define load line in transistor characteristics.

- What are the salient features of ‘‘h’’ parameters?

- An amplifier has a voltage gain of 1000 and a feedback ratio of 0.001. Find the gain with feedback.

- What are the two important conditions for oscillations?

- Define Slew rate in a Op–Amp.

- What is meant by virtual ground in an Op–amp?

- Find the frequency of oscillation of a symmetrical astable multivibrator has C1=C2=2nF and R1=R2=10 K Ohms.

- What is meant by Hysterisis voltage in a Schmitt trigger circuit?

PART B — (5  16 = 80 marks)

- (i)	With neat diagrams explain the formation of PN junction and formation of space charge, field distribution in it.	(8)

- (ii)	Explain the phenomenon of rectification in a PN junction.	(8)

- (a)	(i)	Explain the self bias scheme of establishing stable operating point in transistor under Common emitter configuration.	(8)

- (ii)	For a CE silicon transistor amplifier circuit, the bias is provided by potential divider arrangement (self bias). The parameters are 
Vcc = 12 V, Rc = 2.7 K Ohms, RE = 1.8 K Ohms, R1= 56 K Ohms, R2= 18 K, β = 100. Determine the operating point.	(8)

Or

- (b)	(i)	Deduce the H parameter equivalent circuit of a Common Base transistor amplifier and define the parameters.	(8)

- (ii)	Explain the high frequency (Pi) model for a transistor.	(8)

- (a)	Explain the voltage series feedback amplifier with an example. Also derive the expression for voltage gain, input resistance and output resistance.

Or

- (b)	Explain the operation of a Hartley oscillator and derive the expression for oscillation frequency.

- (a)	(i)	What are the characteristics of an ideal operational amplifier.	(4)

- (ii)	Explain the operation of an Op–Amp with a Block diagram.	(12)

Or

- (b)	(i)	With a neat circuit explain the working of an instrumentation amplifier and hence deduce the expression for gain.	(8)

- (ii)	Explain the Op–Amp integrator with necessary diagrams.	(8)

- (a)	With neat circuit diagram and waveforms discuss the operation of a transistor based astable multivibrator.

Or

- (b)	(i)	Briefly explain the features of a 555 IC timer.	(4)

- (ii)	Explain the functional diagram of a 555 IC timer.	(12)

——————