ANNA UNIVERSITY, CHENNAI 25

MODEL QUESTION PAPER

B.E. MECHATRONICS  ENGINEERING, VI SEMESTER

# EE351 - Digital Signal Processing

TIME – 3 Hours						MAXIMUM : 100 Marks

PART – A 		      (10 x 2 = 20 Marks)

ANSWER ALL QUESTIONS

- What are the advantages of digital signal processing?

- Sketch two commonly used discrete signals in DSP

analysis and design?

- Is the following system invertible?

y(t) = x2(t)

- The Z-transform of a particular sequence is given by

z + 1

F(z) =

(1 + z2)z

Obtain the initial and final value of f(n).

5	Distinguish between linear and circular convolution

- Calculate the number of multiplications needed in the calculation of

DFT and FFT with 64-Point sequence.

## A 50Hz sinusoidal  signal is sampled at 30Hz.

What is the lowest aliasing frequency?

## Show that a shift in time results in multiplication by z upon

## Z-transformation.

- What are the disadvantages of FIR filter over an IIR filter?

10	Explain the concept behind use of window functions on digital filter design.

# PART – B 		(5 x 16 = 80 Marks)

11. (i) Test whether the system described by the equation

y[n] = x[n] cos w0[n] for

  - Linearity and 							(4)

(b)   shift invariance.						(4)

(ii)  Verify the causality of the following system

y[n] = x[-n]								(4)

(iii) Find the inverse z-transform of X(Z)  using contour

integral method							(4)

when X[z] = 1/ (1-az-1) ; IzI a.

12(a) (i) Perform the circular convolution of the following sequence.

x1[n] = [1,1,2,1]

### x2[n] = [1,2,3,4]							(8)

(ii) Find the Fourier Transform of the signal shown below

(8)

+1

t

-1

( OR )

12 (b) Compute the 8 point DFT of the sequence

x[n] =   1  for   0  n  7

0, otherwise 	 					(16)

using DIF algorithm

13 (a) Design a digital band pass Butterworth filter to suit the  following

specifications using Bilinear transformation.

Sampling frequency =8kHz

Attenuation of 2 dB or less in the pass band 800Hz < f  < 1000Hz

Attenuation of 20 dB or more in the stop band

0 < F < 400Hz and 2000 < f <    					(16)

( OR )

  - Discuss a digital band pass Chebyshev filter to suit the

specification stated in 13(a) .						(16)

14 (a)  Design a digital HPF to match the following frequency response

### Hd(ejw) = { 1 	/4  IwI 

### { 0		 IwI  /4.

using a  hamming window with w = 11.					(16)

( OR )

14(b) (i) Obtain a cascade realization of the system characterized by the

transfer function

2 ( z + 2 )

H(z)  =  -------------------------------

z ( z-0.1) (z+0.5)(z+0.4) 				(6)

(ii) Determine the parallel realization of the IIR digital filter transfer function

3(2 z 2 +  5 z  + 4)

H(z)  = -------------------------------

(2 z + 1)(z + 2) 					(6)

- Compare the relative merits and demerits of the realizations obtained

in 14 b(i) and 14 b(ii).							(4)

15  (a)  Write a detailed note on

(i)  Need for multirate signal processing				(8)

(ii) 	Identities used in multirate processing.				(8)

( OR )

(b) (i) Explain the ECHO cancellation in telephone Networks with

neat block diagram.							(8)

(ii) Write a detailed note on FM Stereo Applications			(8)