M 091

MODEL PAPER

B.E. DEGREE EXAMINATION.

Fourth Semester

Production Engineering

PR 238 — FLUID POWER DRIVES AND CONTROLS

Time : Three hours	Maximum : 100 marks

Answer ALL questions.

PART A — (10  2 = 20 marks)

- Compare the  hydraulic and pneumatic systems.

- Draw the symbols of the following :

- (a)	5/2 direction control valve–pilot operated and spring return

- (b)	Pressure relief valve.

- Distinguish between the positive and non–positive displacement pumps.

- Why the volumetric efficiency of a positive displacement must be rated with pressure?

- What is an FRL unit and mention its use?

- Name any four ways of mounting the cylinders.

- What is a Latch circuit?

- What is a Coanda Effect?

- Distinguish between PLC and computer.

- What is a travel step diagram?

PART B — (5  16 = 80 marks)

- (i)	Discuss the merits of Fluid power system and compare it with other drive system.				(4)

- (ii)	Metal sheets are to be flanged on a pneumatically operated bending tool. After clamping the component by means of a single acting cylinder A, it is bent over by a double acting cylinder B and subsequently finish bent by another Double acting cylinder C. The operation is initiated by a manual push button.

- Auxillary conditions :

- (1)	Only one working cycle is to be completed each time a start signal is 
	given.

- (2)	Cylinder B for bending operation may only advance when clamping 
	cylinder A has reached its from end position and the required 
	clamping pressure is available, design a suitable circuit.

- Positional sketch :

- (a)	(i)	How pumping action is accomplished in a positive displacement 
	pump?			(6)

- (ii)		Describe the principle, operation and construction of a Vane pump.	
						(10)

Or

- (b)	(i)		What is a hydrostatic drive and explain its advantages?	(4)

- (ii)		Explain with neat sketch the principle of operation of a axial piston 
		pump and also derive an expression when the pump is converted as 
		a motor.			(12)

- (a)	(i)	What is a counter balance valve? Explain its significance.	(6)

- (ii)		Explain with neat sketch the construction and operation of a 
		Balanced piston pressure relief valve.	(10)

Or

- (b)	(i)		List down the different types of pressure control valves.	(4)

- (ii)		What is a accumulator and explain the working details of any two 
		types? Sketch any two–accumulator circuits.

- (a)	Design a sequencing circuit for the sequence shown below by Cascade method A+ A− B+ B− C+ C− where + stands for forward stroke 
and – stands for return stroke of the cylinder.

Or

- (b)	(i)		What is the Hi–Lo circuit? Explain its use.	(8)

- (ii)	Distinguish meter in, meter out and bleed off circuits. Explain their 
	selection criteria.			(8)

- (a)	Design and draw a electro pneumatic sequencing circuit for the sequence A+ B+ B− A− by Karnaugh vetich map method.

Or

- (b)	(i)		Draw a electro pneumatic ladder diagram for A + A− B+ B− .	(10)

- (ii)		Modify the above electro pneumatic ladder diagram for a PLC.	(6)

—————