M 015

MODEL PAPER

B.E. DEGREE EXAMINATION.

Second Semester

Computer Science and Engineering

PH 136 — SEMICONDUCTOR PHYSICS AND OPTO ELECTRONICS

Time : Three hours	Maximum : 100 marks

Answer ALL questions.

PART A — (10  2 = 20 marks)

- Calculate the number of atoms belonging to a unit cell of a face centered  cubic (FCC) structure.

- What are the desired characteristics of a photoconductive material?

- What is Meissner effect?

- What are the different applications of metallic glasses?

- What is meant by electro absorption?

- What is the general principle of a Self–Electro–optic Effect Device?

- What are the conditions for total internal reflection to occur at the fiber walls?

- What is soliton?

- What are the advantages of CD?

- What are required to record a hologram?

PART B — (5  16 = 80 marks)

- (i)	Explain the processes ‘Analog Modulation’ and ‘Digital Modulation’ with figures.				(4 + 4)

- (ii)	Describe the construction and working of a electro optic amplitude modulator.				(8)

- (a)	Derive an expression for density of states and hence calculate the carrier concentrations in metals and semiconductors.	(16)

Or

- (b)	(i)	What are the advantages of photoconductive mode?	(2)

- (ii)	Describe silicon photodiode structure for photoconductive operation.
				(10)

- (iii)	Write the main factors that limit the speed response of photodiodes.
				(4)

- (a)	(i)	Describe the action of LCD display device in reflection mode.	(6)

- (ii)	Describe two methods of connecting an array of display elements.	(6)

- (iii)	What are the advantages and disadvantages of LCDs?	(4)

Or

- (b)	(i)	Why electronic chip has to be packaged properly at various stages before its use?			(4)

- (ii)	Draw and explain the structure of a typical chip carrier.	(8)

- (iii)	What are Multilayer Multi Component substrates?	(4)

- (a)	(i)	What are single mode and multi mode fibers?	(4)

- (ii)	Derive an expression for numerical aperture of the fiber.	(8)

- (iii)	Calculate the acceptance angle of a fiber with a core index of 1.54 and a cladding index of 1.5.		(4)

Or

- (b)	(i)	Explain with a schematic diagram  an optical communication system.			(6)

- (ii)	What is attenuation and how signal amplification is done in fibers?	
				(6)

- (iii)	Write different attractive features of fiber optic communication.	(4)

- (a)	(i)	Explain the design and data storage system of a floppy disk and a magnetic tape.			(5 + 5)

- (ii)	Explain how the CDs are classified based on their storage technique and capabilities.			(6)

Or

- (b)	(i)	What is holographic storage of information?	(2)

- (ii)	What happens to the hologram when cut into pieces?	(2)

- (iii)	Explain the construction and reconstruction of a hologram with neat sketches.			(6 + 6)

———————